=============================================================================

List of generated methods in project: sci_echo

THIS TEXT DESCRIPTION IS GENERATED BY THE TOOL. DO NOT MODIFY IT.

=============================================================================

Module "AS1"       (bean AsynchroSerial)

- AS1_RecvChar -If any data is received, this method returns one character, otherwise it returns an error code (it does not

wait for data). This method is enabled only if the receiver property is enabled.  [Note:] Because the preferred

method to handle error and break exception in the interrupt mode is to use events <OnError> and <OnBreak> the

return value ERR_RXEMPTY has higher priority than other error codes. As a consequence the information about an

exception in interrupt mode is returned only if there is a valid character ready to be read. Version specific

information for Freescale 56800 derivatives ]   DMA mode:  If DMA controller is available on the selected CPU

and the receiver is configured to use DMA controller then this method only sets the selected DMA channel. Then

the status of the DMA transfer can be checked using GetCharsInRxBuf method. See an example of a typical usage

for details about the communication using DMA.

- AS1_SendChar -Sends one character to the channel. If the bean is temporarily disabled (Disable method) SendChar method

only stores data into an output buffer. In case of a zero output buffer size, only one character can be stored.

Enabling the bean (Enable method) starts the transmission of the stored data. This method is available only if

the transmitter property is enabled.  Version specific information for Freescale 56800 derivatives ]   DMA mode:

If DMA controller is available on the selected CPU and the transmitter is configured to use DMA controller then

this method only sets selected DMA channel. Then the status of the DMA transfer can be checked using

GetCharsInTxBuf method. See an example of a typical usage for details about communication using DMA.

- AS1_RecvBlock -If any data is received, this method returns the block of the data and its length (and incidental error),

otherwise it returns an error code (it does not wait for data).  This method is available only if non-zero

length of the input buffer is defined and the receiver property is enabled.  If less than requested number of

characters is received only the available data is copied from the receive buffer to the user specified

destination. The value ERR_EXEMPTY is returned and the value of variable pointed by the Rcv parameter is set to

the number of received characters. Version specific information for Freescale 56800 derivatives ]   DMA mode:

If DMA controller is available on the selected CPU and the receiver is configured to use DMA controller then

this method only sets the selected DMA channel. Then the status of the DMA transfer can be checked using

GetCharsInRxBuf method. See an example of a typical usage for details about communication using DMA.

- AS1_SendBlock -Sends a block of characters to the channel.  This method is available only if non-zero length of the output

buffer is defined and the transmitter property is enabled.  Version specific information for Freescale 56800

derivatives ]   DMA mode:  If DMA controller is available on the selected CPU and the transmitter is configured

to use DMA controller then this method only sets the selected DMA channel. Then the status of the DMA transfer

can be checked using GetCharsInTxBuf method. See typical usage for details about communication using DMA.

- AS1_ClearRxBuf -Clears the receive buffer.  This method is available only if non-zero length of the input buffer is

defined and the receiver property is enabled. Version specific information for Freescale 56800 derivatives ]

DMA mode:  If DMA controller is available on the selected CPU and the receiver is configured to use DMA

controller then this method only stops selected DMA channel.

- AS1_ClearTxBuf -Clears the transmit buffer.  This method is available only if non-zero length of the output buffer is

defined and the receiver property is enabled. Version specific information for Freescale 56800 derivatives ]

DMA mode:  If DMA controller is available on the selected CPU and the transmitter is configured to use DMA

controller then this method only stops selected DMA channel.

- AS1_GetCharsInRxBuf -Returns the number of characters in the input buffer. This method is available only if the receiver

property is enabled. Version specific information for Freescale 56800 derivatives ]   DMA mode:  If DMA

controller is available on the selected CPU and the receiver is configured to use DMA controller then this

method returns the number of characters in the receive buffer.

- AS1_GetCharsInTxBuf -Returns the number of characters in the output buffer. This method is available only if the

transmitter property is enabled. Version specific information for Freescale 56800 derivatives ]   DMA mode:  If

DMA controller is available on the selected CPU and the transmitter is configured to use DMA controller then

this method returns the number of characters in the transmit buffer.

Module "Cpu"       (bean 56F8006_48_LQFP)

- Cpu_EnableInt -Enables all maskable interrupts

- Cpu_DisableInt -Disables all maskable interrupts

- Cpu_SetWaitMode -Sets low power mode - Wait mode.  For more information about the wait mode see this CPU documentation.

Release from wait mode: Reset or interrupt

- Cpu_SetStopMode -Sets low power mode - Stop mode.  For more information about the stop mode see this CPU documentation.

===================================================================================