ADC Average driver using the IO Processor (IOP) in the MPC5510 Family

- Introduction

In real-world applications there are occasions when it may be inappropriate to use a single ADC conversion result. Although the result itself is a correct conversion of

the analog value that existed at the input, it may be that an external disturbance has caused it to be unrepresentative of the signal of interest.

This is particularly of concern where a short series of results is disturbed by some undesired noise at the input.

One method of addressing this problem, and of rejecting the disturbance, is to average the incoming ADC values and so extract the longer-term value of the signal. This is most effective when the desired signal changes slowly.

The averaging can be applied over a number of samples, and the output based on the most recent results. Each time a result is taken from the ADC module, the oldest previous result is deleted, and the average is recalculated using the remaining results and the newest value.

- Principle of Operation

The ADC Average driver based its operation over the principle of a Boxcar Filter. The Boxcar filter is a simple digital filter with a finite impulse response (FIR) and its principle is used to create an output series composed by the runtime mean of input series values.

The simplicity of a Boxcar filter is due to the fact that each sample is multiplied by a unit-valued coefficient that performs a filtering task without multiplies. The overhead of this filter is from the typical “multiply and accumulate” to simply “accumulate”.

After sample accumulation is done, the final step in the FIR is to normalize all samples by dividing the calculated result by the sum of filter coefficients, which, in this case is simply the number of samples to be averaged.

Figure 1.0 shows the impulse response of a Boxcar Filter.

h(n)

n

Figure 1.0. Impulse Response of Boxcar Filter

Figure 2.0 shows the frequency response of a Boxcar Filter using 8-step configuration. The frequency responses of this type of filter gives a general low-pass characteristic, but with lots of lobs (see Figure 2.0).

This has the effect of preserving the desired, slowly changing (low frequency) signal in the presence of sudden changes (high frequency noise). It is important to bear this frequency response in mind when considering the type of noise that the averaging filter can reject.

Figure 2.0: Frequency Response of 8-step Boxcar Filter

Figure 3.0 describes the mathematical principle of a Boxcar filter is:

y(n)=i=0Pbix(n−i)

Figure 3.0: Mathematical expression of a Boxcar filter.

- ADC Average Configuration and Control Parameters.

This Section describes the way to configure the ADC Average driver to operate during run-time and describes pre-compile statements as well.

  - Pre-Compile Definitions.

The Adc Average driver has many pre-compile definitions to enable/disable certain functionalities in the driver. All pre-compile definitions are located in file named “AdcFltr_Cfg.h” with the exception of macro named “SYS_FREQ” which is located in “System_Cfg.h” file.

3.1.1 SYS_FREQ

This definition describes the operating system frequency of the whole driver. The ADC Average driver uses this definition to calculate the timing references of the PIT channel used to manage the task scheduler. This field must be captured at Hz units as shown below:

Example: #define SYS_FREQ  (uint32_t) 64000000U  /* Hz units */

    - ATDFLTR_0_AVERAGECHANNELS

This macro defines the number of ADC channels that the software will average. The value of this macro must be between 1, the maximum number of ADC channels allowed in the MPC5510.

Example:  #define ATDFLTR_0_AVERAGECHANNELS      6U

    - ATDFLTR_0_AVERAGETAPS

This macro defines the number of values to include in the average calculation. It basically indicates to the ADC Average driver how many samples shall be used to calculate the final average value per ADC channel.

The driver allows only certain predefined average taps which are validated by the Adc Average Error layer (described in section x.x). The tap ranges allowed by the driver are: 2, 4, 8, 16, 32 and 64.

Main reason why only these ranges are allowed by the driver is to avoid using the division instruction. Instead the driver shifts to the right the number of positions indicated by the macro (ATDFLTR_0_AVERAGETAPS). This feature provides a better performance to the driver.

These tap ranges are represented by the follow macros described below:

#define _2_SAMPLE_DEPTH_         1U

#define _4_SAMPLE_DEPTH_         2U

#define _8_SAMPLE_DEPTH_         3U

#define _16_SAMPLE_DEPTH_        4U

#define _32_SAMPLE_DEPTH_        5U

#define _64_SAMPLE_DEPTH_        6U

To set the number of taps (or samples) that the driver uses to calculate the average value, user must first set the macro named SAMPLE_AVERAGE_VALUE with one of the above described parameters.

Example:  #define SAMPLE_AVERAGE_VALUE      _8_SAMPLE_DEPTH_

During run time, the ADC Average driver shall use the ATDFLTR_0_AVERAGETAPS macro definition to indicate the number of taps that the driver averages per ADC channel.

Example:  #define ATDFLTR_0_AVERAGETAPS  SAMPLE_AVERAGE_VALUE

    - ATDFLTR_0_AVERAGEINTERRUPT

When defined as TRUE (ON state), this macro causes the driver to raise an interrupt by invoking the function previously configured in the macro named ATDFLTR_0_FILTER_FNC.

Example:  #define ATDFLTR_0_AVERAGEINTERRUPT ON

Example:  #define ATDFLTR_0_FILTER_FNC &vfnFilter_1_End

    - ADC_FLTR_ERROR_DETECT

When defined as TRUE (ON state), the ADC Average Error Layer enables the validation of parameters passed throughout the ADC Average functions.

Example: #define ADC_FLTR_ERROR_DETECT   ON

  - Run-time parameters.

The run-time parameters in the ADC Average driver helps to track the information about the number of samples the driver retrieves per ADC channel. Also, these run-time parameters contain the buffers to store each of the samples per ADC channel.

3.2.1 ADC Average Control Structure

typedef struct

{

uint16_t numTAPS:6;

uint16_t numChannels:6;

uint16_t interruptCPU:1;

void (*AverageEndFnc)(void);

} tAdcFltr_ChannelConfig;

numTAPS:      Contains the number of samples to be taken for each ADC channel.

numChannels: Indicates the total number of ADC Channels to be converted.

interruptCPU: Causes the driver to jump into a predefined function to indicate the driver has averaged the configured ADC channels.

3.2.1 ADC Average Channel Structure.

typedef struct

{

uint16_t*  pSampleBuffer;

vuint16_t* pResultRegister;

uint16_t   u16AccumulateSample;

uint16_t*  pFilterResult;

} tAdcFltr_ChannelStruct;

pSampleBuffer: Pointer to the stored ADC samples. This pointer is used by the driver to store the samples per ADC channel.

pResultRegister: Pointer to the ADC result variable handled by the eDMA module. eDMA module stores the ADC results in the rQUEUE array.

u16Accumulate: This variable accumulates the sample results retrieved per ADC channel.

pFilterResult: Pointer to the Averaged results from every configured ADC Channel.

- Required Resources to Operate the ADC Average driver.

Table 1.0 show the on-chip resources required to operate the ADC Average driver.

| Parameter | Value |
| --- | --- |
| Mandatory peripheral use | ADC channels as required |
| Mandatory peripheral use | eDMA channels 0 & 1 required |
| Optional peripheral use | PIT channel |

Table 1.0: ADC Average required resources

The driver requires the availability of ADC channels to perform an Average of each sample taken from ADC channels. If a specific sampling frequency is required then the ADC Average driver must make use of any available PIT channel.

- Driver Implementation.

  - ADC Average Initialization.

The driver contains an initialization routine which initializes the control parameters of the driver and configures the ADC module to operate at certain conversion frequency, bit resolution, etc.

This function called “vfnAdcFltr_Init” initializes the Filter result registers in which the average value of each ADC channel is stored, the sample buffer pointer and the pointer where the driver reads the ADC converted value.

The ADC Average initialization function also validates that the configuration pointer structure passed through the function has valid parameters.

The second parameter to validate by the function is the number of samples to be converted for each ADC channel. If the function detects an invalid condition outside of the allowed ranges, the initialization flow aborts immediately leaving the ADC registers and buffers uninitialized. The allowed sample ranges are: 2, 4, 8, 16, 32 and 64.

After the initialization function validates correct configuration parameters, the control buffers are initialized and the ADC module is configured based on the parameters located in the file called “Adc_Cfg.h”

  - ADC Average Driver Configuration.

The ADC Average driver operates based on a few configuration parameters located in the file named “AdcFltr_Cfg.h”.

These parameters are the ADC channels to be averaged, the number of samples taken per ADC channel, the total number of ADC channels to be averaged and the function to jump into once the driver finishes the averaging. The procedure to configure the driver is explained below:

- Configuring the ADC Channels needed to be averaged. These channels shall be indicated in the configuration array located in the file named “AdcFltr_Cfg.h”.

Example:

uint16_t u16AdcChannel_Convert[ATDFLTR_0_AVERAGECHANNELS] =

{

ADC_CHANNEL_0,  /* Adc Channel 0 */

ADC_CHANNEL_1,  /* Adc Channel 1 */

ADC_CHANNEL_2,  /* Adc Channel 2 */

ADC_CHANNEL_3,  /* Adc Channel 3 */

ADC_CHANNEL_4,  /* Adc Channel 4 */

ADC_CHANNEL_5   /* Adc Channel 5 */

};

- The total amount of ADC channels to be averaged.

Example: #define ATDFLTR_0_AVERAGECHANNELS   6

- The number of samples to be averaged by the driver.

Example: #define SAMPLE_AVERAGE_VALUE   _8_SAMPLE_DEPTH_ /* 8-Step samples */

- Enable or Disable the End Filter interrupt function. This condition indicates the driver to jump into a specific predefined function when the channel averaging is done.

Example: #define ATDFLTR_0_AVERAGEINTERRUPT     ON

#define ATDFLTR_0_FILTER_FNC           &vfnFilter_1_End

Figure 4.0 shows the ADC Average configuration structure.

tAdcFltr_FilterStruct sATD0 =

{

AdcFltr_0_ChannelStruct,

{

ATDFLTR_0_AVERAGETAPS,

ATDFLTR_0_AVERAGECHANNELS,

ATDFLTR_0_AVERAGEINTERRUPT,

ATDFLTR_0_FILTER_FNC

}

};

Figure 4.0: ADC Average Configuration Structure.

  - ADC Averaging.

The driver makes use of the function called “vfnAdc_Sample_Acquire” to trigger the ADC channels to be sampled. The function triggers the eDMA module to start sending commands to the ADC module and to read the converted results from the ADC module.

Every time this function is invoked, the eDMA send commands to convert 4 ADC Channels at the same time. This feature provides a better performance to the driver when many ADC channels being requested to be averaged.

The ADC averaging function figures it out to send commands to the ADC module when less than four channels are configured to be averaged. The function uses a circular command buffer to store conversion commands. If only two ADC Channels needs to be averaged, the function fills the rest of the two command buffers with the same commands from first ADC channels.

If a specific sampling frequency is required, a PIT channel shall be initialized to invoke this function (“vfnAdc_Sample_Acquire”) periodically.

- Sample Application

6.1 Introduction

Sample application is provided within this application note to demonstrate the operation of the ADC Average driver. A Multi-Task scheduler has been mounted in the sample application to retrieve the averaged results.

6.2 Initialization

Figure 7.0 shows the sample application flow chart.

Enable ADC mode on pins 0, 1 ,2 and 3.

Configure Bus Clock to run up to 64MHz

Configure eDMA channel 0 to transfer conversion commands from cQUEUE buffer to ADC

Configure eDMA channel 1 to transfer conversion results to rQUEUE buffer

Configure ADC0 Module

ADC Average driver

Configure PIT Channel 2 to be serviced every 100us

Configure and Initialize ADC Average driver

Configure Scheduler driver using PIT Channel 1 as Time reference

Start Scheduler Execution

1

Scheduler driver

1

10ms elapsed?

No

Yes

Read ADC Average Filter Result

Store result values into “u16Avg_Channel” buffer array.

2

2

Figure 7.0: Sample Application Flow Chart

The very first action the sample application performs is to configure the microcontroller bus clock to run up-to 64MHz. After the bus clock is set, the microcontroller pins number 0, 1,2 and 3 are configured to enable the ADC mode.

When ADC functionality pins are enabled, the eDMA channels 0 and 1 are configured to handle transfers and store conversions from the ADC module.

eDMA channel 0 is used to handle conversion commands transfers to the ADC module and eDMA channel 1 is configured to store result conversions. A Conversion command buffer called “cQUEUE” is used to transfer commands from buffer to ADC module and a result conversion buffer called “rQUEUE” is used to store converted results from ADC channels.

When the eDMA channels are properly configured, the ADC initialization is executed to start the configuring the general parameters of the ADC module such as the ADC conversion rate, bit resolution and result alignment. All these parameters are taken from the ADC configuration file named “Adc_Cfg.h”.

The PIT channel 1 is enabled to be serviced every 100us.  Every 100us the ADC Averaged function is invoked to start averaging the configured ADC Channels. This interrupt time base provides a frequency conversion rate of 10KHz.

The last initialization step executed is the configuration of the ADC Average routine. The routine configures the “cQUEUE” buffer to indicate the ADC module that channels 0, 1 , 2 and 3 needs to be converted. The follow configuration parameters are set to operate with this sample application:

- Maximum number of ADC channels equals to 4.

- Number of Samples per ADC channel equals to 8.

- The driver shall generate an interrupt after all ADC channels being averaged. This condition is true whenever the interrupt macro in the driver is enabled.

Figures 5.0 shows the configuration parameters used in the sample application.

uint16_t u16AdcChannel_Convert[ATDFLTR_0_AVERAGECHANNELS] =

{

ADC_CHANNEL_0,  /* Adc Channel 0 */

ADC_CHANNEL_1,  /* Adc Channel 1 */

ADC_CHANNEL_2,  /* Adc Channel 2 */

ADC_CHANNEL_3  /* Adc Channel 3 */

};

/* Filter struct     */

tAdcFltr_FilterStruct sATD0 =

{

AdcFltr_0_ChannelStruct,

{

ATDFLTR_0_AVERAGETAPS,

ATDFLTR_0_AVERAGECHANNELS,

ATDFLTR_0_AVERAGEINTERRUPT,

ATDFLTR_0_FILTER_FNC

}

};

/** Indicates the driver the number of samples to be acquired per channel          */

#define SAMPLE_AVERAGE_VALUE      _8_SAMPLE_DEPTH_

/** 1..8/16 ; number of active channels used on this ADC            */

#define ATDFLTR_0_AVERAGECHANNELS      6U

/* 1..64 ; size of filter - shared among all filters on this ADC    */

#define ATDFLTR_0_AVERAGETAPS          SAMPLE_AVERAGE_VALUE

/** If TRUE then XGATE interrupts CPU when done                     */

#define ATDFLTR_0_AVERAGEINTERRUPT     ON

/** Filter End function used to indicate that all samples were averaged */

#define ATDFLTR_0_FILTER_FNC           &vfnFilter_1_End

/** Enables or Disables the Error Layer                             */

#define ADC_FLTR_ERROR_DETECT          ON

Figure 5.0: ADC Average Configuration Parameters.

5.3 Scheduler Initialization

After initialization execution, the scheduler initialization function named “vfnScheduler_Init” runs to configure the PIT channel number 2 as a time base reference.

A macro definition named “LOOP_TIME_5ms” located at “Pit.h” file calculates the value of the PIT channel 2 counter to generate an interrupt service every 5 ms. This macro makes use of the actual system clock used by the sample application which is 64 MHz.

After the “vfnScheduler_Init” function is invoked, the scheduler execution remains in hold state until the function named “vfnStart_Scheduler” starts the execution of the scheduler. This function enables the PIT channel 2 timer to start counting and enables the complete operation of the scheduler.

Listing 2 shows the correct initialization and execution of the Scheduler.

vfnScheduler_Init();  /* Initialize Scheduler timebase         */

vfnStart_Scheduler(); /* Start Tasks execution                 */

Listing 2. Schedule initialization procedure.

IMPORTANT

It is important to remark that the Scheduler driver is not part of the ADC Average driver. Scheduler driver helps to demonstrate the functionality of the ADC Average driver.

5.4 Sample Application

Every 10 ms the scheduler reads the ADC Average Filter result buffer to retrieve the values averaged from channels 0, 1 ,2 and 3. This process is executed every 10ms and the process is mounted under a function-like macro named “EXECUTE_10MS_TASKS”.

Figure 6.0 shows the scheduler execution and shows the 10ms thread invoked to retrieve the averaged channel values.

while (gu8SleepModeEnabled == 0)

{

if ((gu8Scheduler_Flag & (uint8_t)0x01) == (uint8_t)0x01)

{

/*-- Allow 10 ms periodic tasks to be executed --*/

EXECUTE_10MS_TASKS();

/* Scheduled tasks finished, clear control flag */

gu8Scheduler_Flag = (uint8_t)0x00;

}

else

{

if ((gu8Scheduler_Flag & (uint8_t)0x02) == (uint8_t)0x02)

{

/*-- Allow 20 ms periodic tasks to be executed --*/

EXECUTE_20MS_TASKS();

/* Scheduled tasks finished, clear control flag */

gu8Scheduler_Flag = (uint8_t)0x00;

}

else

{

if ((gu8Scheduler_Flag & (uint8_t)0x04) == (uint8_t)0x04)

{

/*-- Allow 40 ms periodic tasks to be executed --*/

EXECUTE_40MS_TASKS();

/* Scheduled tasks finished, clear control flag */

gu8Scheduler_Flag = (uint8_t)0x00;

}

else

{

if ((gu8Scheduler_Flag & (uint8_t)0x08) == (uint8_t)0x08)

{

/*-- Allow 80 ms group A periodic tasks to be executed --*/

EXECUTE_80MS_A_TASKS();

/* Scheduled tasks finished, clear control flag */

gu8Scheduler_Flag = (uint8_t)0x00;

}

else

{

if ((gu8Scheduler_Flag & (uint8_t)0x10) == (uint8_t)0x10)

{

/*-- Allow 80 ms group B periodic tasks to be executed --*/

/* Scheduled tasks finished, clear control flag */

gu8Scheduler_Flag = (uint8_t)0x00;

}

}

}

}

}

}

The averaged results are stores in the buffer variable named “u16Avg_Channel”. The buffer array contains the result from the four configured ADC Channels.