- Resistive Touchscreen Controller using the S08 Family

by:	Andy Juengst

Field Applications Engineer

Revision: 0.1 – December 1, 2009

Touchscreen Controller using S08

1	Introduction

2	Resistive Touchscreen Fundamentals

2.1	4-Wire Touchscreen Interface

2.2	8-Wire Touchscreen Interface

2.3	5-Wire Touchscreen Interface

3	Resistive Touchscreen Application Considerations

3.1	Conversion Resolution

3.2	Environment Impact

3.2.1	Noise

3.2.2	Coordinate Median Filtering

3.2.3	Sampling Delay

3.2.4	Pen Up/Down Debounce

3.3	Calibrations

3.3.1	Resistive Losses Calibration

3.3.2	Touchscreen / Display Misalignment Calibration

3.3.3	Normalization

3.4	Coordinate Refresh Rate

4	Communication

4.1	SCI

4.2	Communication Protocol

4.2.1	Transmitting

4.2.2	Receiving

5	MC9S08QG8 Pin-outs

6	Sample Code Flow chart

7	Reference material

8	Acronyms

  - Introduction

Several materials available today have the unique property of being both transparent at optical frequencies and electrically conductive.  Because of these properties, these materials can be placed over display panels and used to “select” options or “interact” with graphics displayed on the panel.  Pressing or touching a point on the screen changes the electrical properties of the material that can be detected by a microcontroller.  Several methods are available to detect this touch.  Capacitive touch senses the change in charge rate caused by touching the panel.  This method is out of scope for this app note and will not be discussed.  You can find more detail about capacitive touch sensing at www.freescale.com/proximity.  The more common method is to use two layers of the resistive material separated by a small distance. When pressed, the two layers short and form a voltage divider that can be measured.   There are 4-wire, 5-wire, and 8-wire resistive touch panels.  This app note will describe all 3 methods.  A software example is provided for the 4 and 8 wire methods.  Modification to a 5-wire panel would only require a minimal amount of software rework.  The sample code can be found on the same web page as this app note.

  - Resistive Touchscreen Fundamentals

    - 4-Wire Touchscreen Interface

X+

X-

Y+

Y-

Point of touch

X-

Y+

Y-

X+

Figure 1: Schematic of 4-Wire Touchscreen

Figure 1 shows the basic make-up of a 4-wire touchscreen interface.  A piece of transparent conductive material has contacts placed on opposite ends.  A second piece of the same material, with contacts oriented 90 degrees to the first, is placed over the first piece with a small gap between the two.  By pressing on the film, the two layers are made to come in contact with each other.  The result is a simple voltage divider that can be sensed with a microcontroller.  For example to measure the Y-coordinate: Y+ is driven to Vdd and Y- is driven to Vss.  X- is set to an input with no pull up/down resistor, which effectively takes it out of the circuit.  X+ is connected to an Analog-to-Digital input.  A reading on X+ will give the voltage at the y-position of the pen-down point.  The voltage ratio of this point to Vdd is the same as the A/D reading to the A/D range.  This setup is shown in Figure 2a.  The reading yields how far up the y-axis the point is by the following ratio:

ADreadingADresolution=yvalueheightofpanel	Equation 1

“A/D resolution” is the full-range of the A/D mode depending on how many bits of resolution the A/D is set for (i.e. A/D resolution = 4096 for 12-bit mode.)

“Height of panel” can be any scale the system requires and will be a known reference value to the host microprocessor.  One common ‘height’ is to use the full scale A/D resolution.  The measured A/D reading then becomes the transmitted value to the host microprocessor, no multiplication or division required.  The host processor knows the ratio of pixels to A/D points and can then convert the reading into the appropriate pixel count from the edge of the panel.  Another common method is to use the resolution of the LCD panel for the ‘height’.  Consequently, the touchscreen controller will send the host processor the pixel data without any further calculations needed on its part.  A similar process is done to determine the x-coordinate.

Most systems do not need the uC continuously reading the control panel.  This is an inefficient use of power since an active “pen-down” condition happens a small percentage of the time.   The setup in Figure 1 also allows for the uC to go into a sleep mode and use an active button press to wake the uC and cause it to start reading data points.  Figure 2b shows the configuration needed to sense a new pen-down condition.  X+ is changed from an A/D pin into a KBI (KeyBoard Interrupt) with the weak internal pull-up resistor enabled.  X- and Y+ are set up as inputs with no pull up/down enabled.  Y- is set as an output driven to Vss.  The uC is put into a low power sleep mode with the KBI enabled on the falling edge of X+.  When a pen-down condition occurs, the resistance to Vss thru the Y- pins is low enough to overcome the weak pull-up on X+.  As a result, X+ is pulled low and a KBI interrupt occurs waking the uC and allowing readings to take place.  Typical operation is to continuously read data points until the pen-up condition is sensed, then go back to sleep.

Y+ =  Vdd

Y- = Vss

X+ = input w/

Weak

pull-up

Figure 2a: A/D Reading

Figure 2b: KBI Wakeup

X+ =  A/D

reading

X- =  floating

input

X- =  floating

input

Y+ =  floating

input

Y- = Vss

A pen down will pull X+ to ground thru VSss

    - 8-Wire Touchscreen Interface

There are inherent losses in the wires, connections, and inactive area of the touchscreen.  As a result, some value less than Vdd and greater than Vss is actually applied to the points noted above causing an error in the conversion.  This error is calibrated out in an 8-Wire touchscreen.  Another Sense pad is placed on each contact as shown in Figure 3.  Vdd is applied to Y+ and Vss to Y-.  A/D readings are taken at both Y+ sense and Y- sense.  The difference between these two readings is now used for the “A/D resolution” value in the Equation 1.  In other words, due to losses in the setup, the full range of A/D values that you might measure for a 12-bit setup will be some value below 4096.  A similar reading is taken for the x-axis.

Y+

Y-

X-

X+

Y- sense

Y+ sense

X+ sense

X- sense

Figure 3: 8-wire Setup

    - 5-Wire Interface

One drawback of these resistive materials is that they can wear off over time.  This wearing can affect the A/D readings as described above.  One method to overcome this degradation is utilized in a 5-wire setup as shown in figure 4.  The bottom layer, which is less susceptible to wear, has pads at each corner.

UL

LL

UR

LR

sense

Figure 4: 5-wire Setup

These pads are labeled UL, UR, LL and LR.  The entire top sheet is the sense pad and scratches and wear will have minimal impact.  In a very similar method as with the 4-wire setup, one can see that the x-coordinate can be determined by applying Vdd on both UR and LR while applying Vss on both UL and LL.  Taking an A/D reading on “sense” yields the x-coordinate.  Similarly, the y-coordinate is obtained by putting Vdd on UL and UR while putting Vss on LL and LR.

Also in a similar manner to the 4-wire setup, the device can be put in a low-power sleep mode and woken up with a key board interrupt.  Three of the pads on the bottom sheet are set up as high-impedance floating inputs.  The 4th is made a KBI with a weak pull-up and “sense” is set to Vss.  When a pen-down condition occurs, the pin set as a KBI will be pulled low to ground thru the “sense” pin forcing a KBI.

  - Resistive Touchscreen Application Considerations

    - Conversion Resolution

The resolution of the touchscreen is determined by the application.  Most applications that are using large “selection buttons” on the display can get by with 8-bit resolution or less.  Applications that use smaller selection buttons, or that have buttons displayed very close or adjacent to each other, may require 10-bit resolution.   For applications that require the capability to follow the pen as it moves around the screen, 12-bit resolution may be required.   The software provided did not test for a moving pen.  The principles used are the same but some modifications may need to be made to sampling rates, filtering, etc. for this type of application.  The sample code is set for 10-bit resolution.

    - Environment Impact

3.2.1    Noise

As anyone who has used analog to digital converters knows, these readings are very susceptible to electrical noise in a system.  Any voltage “bounce” in the power rails, or on the A/D lines, can cause significant error in you’re A/D reading; resulting in the wrong location calculated for the pen-down condition.  As a result, care must be taken to isolate the touchscreen system from any noise source.  In most cases, following good design practices for EMI is enough to obtain good performance.  In extreme environments, isolated power rails and shielded A/D signals may be required.

3.2.2    Coordinate Median Filtering

Even with very aggressive noise abatement techniques, there is always the chance of a spurious noise bounce within a system.  Consequently, many touchscreen applications choose to implement a software filter on the readings.  This application implements a simple Median Filter.  Five consecutive readings are taken and the median value is chosen as the pen-down value.  Depending on application, more (or less) readings, an average, running-average, or more advanced algorithm can be implemented.

### 3.2.3     Sampling Delay

Due to the parasitic capacitance of the system, there can be a delay between configuring the system for a reading and seeing the correct voltage on the A/D pin.  Again the amount of time required will depend on each implementation but for most systems this delay will be quite small and may be ignored.  This application does include a small delay between configuring the ports and starting the reading.  The delay can be increased or eliminated as needed.

Even with the delay, the first reading on a new pen-down condition often results in inaccurate coordinates.  As a result, the sample code does not transmit the first data points.  This results in a 20ms delay between pen-down and the first data transmission.  This is considered small enough for most, if not all, applications.

    - Pen Up/Down Debounce

As mentioned in section 2, a KBI is used to wake the system up from its low-power sleep mode.  This is done by setting a pad on one layer to a KBI with a weak pull-up.  A pad on the second layer is tied to ground.  When a pen-down occurs, the KBI is pulled low to ground thru the second layer.  The KBI controller will typically respond to an interrupting signal within a clock cycle or two.  As a result any noise spike on this pin can wake the system.  A method needs to be implemented to avoid sending false pen down conditions to the main microprocessor.

Similarly, the system needs to detect when a pen-up condition occurs.  This is handled in a similar fashion except that the 1st pad is set to an Input with a weak pull-up (KBI is not required).  If the input is low, a pen-down condition still exists.  A debounce method needs to be implemented to ignore false pen-up conditions.

The sample code provided handles both of these with the same debounce routine.  The code will not actually perform a debounce on a pen down condition prior to entering the main control loop.  When a KBI is sensed, the uC wakes up and reads the coordinates.  It then enters a loop waiting for the timer described in Section 3.4 to expire before transmitting the coordinates.  During the loop, the code continuously performs a pen-up debounce.  When the timer expires, it determines if it has a valid pen-down condition and reacts accordingly:  If pen-up is detected on the first pass thru the code, no data is transmitted and the uC goes back to sleep (a false KBI trigger).  When pen-down is detected, it transmits the measured coordinates and goes back to the top of the loop to measure the coordinates again.  It stays in the loop until a pen-up occurs.  At the end of the loop that detects pen-up, the last valid “pen-down” data points are transmitted and sleep mode is entered.

There are many methods to perform a debounce on an input.  The method that best fits your system requirements should be implemented.  The system chosen in this application is to heavily weight towards a pen-up condition.  While in the debounce loop, the state of the Input pin is monitored.  During this loop, any low value (pen-down condition) will increment a counter.  When a high value (pen-up condition) is read, the counter is reset.  At the end of the transmit interval, when the code leaves this loop, pen-up/down is determined based on the value of this counter.

3.3    Calibrations

3.3.1    Resistive Losses Calibration

Section 2.2 describes how an 8-wire touchscreen can be used to calibrate out the losses in the wiring and pads of the ITO.  The code provided implements an 8-wire touchscreen and performs these calculations every power-up.

3.3.2    Touchscreen / Display Misalignment Calibration

When the resistive layers that comprise the touchscreen are placed over the Display, they often are not aligned, or “squared”, to each other.  This leads to an offset in touchscreen coordinates versus display coordinates.  As a result, an alignment calibration is often needed.  This is the “first time” calibration that many users of touchscreen panels are familiar with.   At the very first power up, the user must pen-down on 3 or 4 different points on the screen.   These known LCD locations are compared with the coordinates from the touchscreen controller to generate a translation routine for all new coordinates coming from the touchscreen controller.

There are a variety of translation routines that can be used.  The below equations are for a simple linear transformation.

Xdisplay=A(Xts)+B(Yts)+CYdisplay=D(Xts)+E(Yts)+F	Equation 2

Xts and Yts are the touchscreen controller’s measured x-y coordinates.  When multiplied by the coefficients, they yield the x-y coordinates of the display, Xdisplay and Ydisplay.  To solve for the six coefficients, A – F, three readings must be taken at known display and touchscreen coordinates, resulting in 6 equations and 6 unknowns.   After solving for and determining the six coefficients, any coordinates measure by the touchscreen controller can easily be converted into display coordinates.

Many of the operating systems; Linux, WinCE®, etc., that are commonly found on the main system processor are already configured to handle this calibration.  They will solve for and apply the coefficients to the raw data that comes from the touchscreen controller.

The sample code does provide for the S08 to perform the translation calculation if desired.  A simple protocol is implemented to receive the six coefficients (12 bytes of data) and store them in NVM.  When this area in Flash is not ‘empty’, the code applies the six coefficients to Equation 2 and sends Xdisplay and Ydisplay to the system microprocessor.  A separate command is available to read and transmit the values stored for the coefficients.  This provides a simple means to verify the touchscreen controller is using the correct coefficient values.  A Reset command is also provided to reverse the process and force the controller to erase the coefficients and send non-translated data, Xts and Yts.  The protocol is defined in section 4.2

The code provided does not have the algorithm to calculate the six coefficients since the main system processor will typically do this.  If you desire the touchscreen controller to perform this calculation, you will need to modify the communication protocol to receive Xdisplay and Ydisplay coordinates.  You will also need to add a function to perform the calculation.  The time for the S08 to perform this calculation will be in the tens of ms.  But since this is a do-once calculation, your system impact will be minimal.

3.3.3    Normalization

When the controller is configured to send Xts and Yts, it is common to normalize the values.  As described in Section 2.1, this is the process of multiplying by the “height of panel” parameter in Equation 1.  Equation 2 can be used to normalize the output data.  By setting coefficients A & E to a normalized value and setting B, C, D and F to zero, the transmitted data is normalized to whatever scale desired.  For example, by setting A & E to 1024 the results are converted back to full 10-bit A/D scale.   Similarly, you can normalize the output to match the resolution of your display.  By setting A = 800 and E = 600, the results will be normalized to a SVGA screen.  The sample code defaults to 1024 x 1024.

### 3.4    Coordinate Refresh Rate

Under a Pen-down condition, the touchscreen controller continuously reads and transmits the coordinates.  How often the coordinate data is sent to the main system processor depends on system requirements.  Too slow and the delayed response will be noticed by the end user.  Too fast and the main processor spends too much time handling unnecessary interrupts - impacting its performance.  The application code provided sends data to the main processor every 10ms.  This timer also controls the main loop in a pen-down condition.  When the timer elapses, data is transmitted to the main system processor and the timer is restarted.  This control time is defined by the variable “TX_interval”, found in ‘control_constants.h’.   The current setup takes 2.5ms to measure and calculate the x-y coordinates, which needs to be taken into account if a faster interval is required.  The BAUD rate of the SCI may also need to be considered if a faster interval is chosen.

- Communication

4.1   SCI

The sample application uses the SCI for communication.  The default BAUD rate is 19.2KBaud.  Code is also in place for 9600baud and 38.4Kbaud.  These values can be selected in the ‘comm.h’ file.  The 9S08QE family also has a SPI and I2C port.  The code is not set up to support these protocols but could easily be added.

The 9S08QE family was chosen because it has the ability to be woken up by the SCI.  This allows for a full two way communication protocol.   The main system microprocessor can send commands at any time and the 9S08QE will wake up and respond.  If this capability is not required in your system, even lower cost 9S08s or 9RS08s may be suitable for your needs.

4.2    Protocol

4.2.1    Transmitting

The uC will send four bytes to the main uP at specific intervals as described in section 3.4.   This protocol should be modified to match your specific system needs.

|  |  | Bit |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Byte | 0 |  |  |  |  | Pen_up |  | Align_c | MinMax_c |
|  | 1 | X7 | X6 | X5 | X4 | X3 | X2 | X1 | X0 |
|  | 2 | Y7 | Y6 | Y5 | Y4 | Y3 | Y2 | Y1 | Y0 |
|  | 3 | Y11 | Y10 | Y9 | Y8 | X11 | X10 | X9 | X8 |

Byte 0:  This is a control byte that provides information to the main uP about the coordinates it is receiving.

Bit 0 – MinMax_c:  This bit tells whether the control provides coordinates that have had the losses due to wiring calibrated out.

0 – Losses are not calculated and removed.

1 – Losses are removed.  (This is automatic in the sample code provided.  Slight modifications need to be made to allow for no calibration.)

Bit 1 – Align_c:      This bit tells whether the control provides coordinates that have calibrated out the misalignment between the touchscreen panel and LCD panel.

0 – Misalignment is not accounted for.

1 – Misalignment is accounted for.

Bit 3 – Pen_up:      This bit indicates a Pen_up condition exists.  In other words, it informs the main uP that this is the last data point being transmitted.

0 – Pen down condition still exists

1 – A pen up condition was detected.  This will be the last transmitted coordinates until a new pen down event happens.

Byte 1:  The lower 8 bits of the X-coordinate.

Byte 2:  The lower 8 bits of the Y-coordinate.

Byte 3:  The lower nibble contains the upper 4-bits of the x-coordinate in a 12-bit resolution system.  Only the lower two bits are used in a 10-bit resolution system.   This nibble is not used for an 8-bit resolution system.  The same situation holds for the upper nibble and the y-coordinate.  The sample code use 10-bit resolution.

4.2.2    Receiving

If the system’s main microprocessor is going to manage the translation calibration as defined in 3.3.2, you may not need to receive any data at all.   SERIAL_RX_SUPPORTED defined in ‘control_constants.h’, allows you to bypass the Receive functions in the main loop resulting in more time de-bouncing the pen-down condition.  You can also move to a lower end S08 or RS08.

The clock source for the SCI is stopped when the 9S08QE is in STOP mode.  The clock takes some time to restart coming out of STOP mode.  As a result, the first incoming byte from the main processor will wake the uC but the data read may be inaccurate.  Consequently, this protocol requires a dummy byte be sent first by the system uP.  The following byte should then be one of the control bytes as described below.

(Note: Once woken, the sample code sits in the RX interrupt routine waiting for the second byte to arrive.  The second byte should come immediately from the main microprocessor.  If there is a significant delay (or you are manually inputting the data into a terminal window), the QE’s internal Watch Dog Timer might expire and RESET the QE.  The QE will restart and function as normal but the transmitting of data from the host controller will need to start over.)

A simple communication scheme is implemented and can be modified as needed by your system.   All bytes are ignored except an ASCII ‘i’, ‘r’, or ‘v’.  ‘r’ will cause the control to delete the stored coefficients and revert to sending non-translated, normalized coordinates.   When an ‘i’ is received, the control waits for 12 more bytes from the main processor.  (Note: Dummy bytes are not needed for the 12 coefficients.  Once the uC sees an ‘i’ it will not allow STOP mode to be entered until all 12 bytes have been received and processed.)  After the 12th byte is received, the control will store the bytes into flash and switch to using the new coefficients when calculating the coordinate to transmit.

The data is sent in the following order:

Byte 0: 	0x69 – initiates the command

Byte 1:	Coefficient A high Byte

Byte 2:	Coefficient A low Byte

.

.

.

Byte 11	Coefficient F high Byte

Byte 12	Coefficient F low Byte

No checksum or error checking is done in the communication protocol; instead the ‘v’ command is implemented.  When the control sees ‘v’, it will transmit the values stored in flash.  The data is sent in the order shown above; starting with Byte 1 up thru Byte 12.  This command not only verifies the communication protocol but also verifies the flash writing routines.

- MC9S08QE8 Pin-outs

Figure 5 below shows the connections between the 16-pin QE8 and an 8-wire touchscreen as used in the sample code.

Pin 6, PTB6, is labeled as a GPIO.  This pin is used in the sample code as an output and controlled by the PTBD_PTBD6 variable.  An oscilloscope was attached to this pin and used for debugging and to measure the timing of the various functions in the code.

Pins 3 & 4 are power and Ground.  Pin 1 is for an external RESET.  It can be used as a general purpose Input if no external RESET is needed.   Pin2 is for Background Debugging and Programming.  This pin can also be used as a general purpose Output.  Pin 5 is another unused GPIO.

1 - /RST

2 – BKGD

3 - Vdd

4 - Vss

5 – PTB7

6 – PTB6

7 – PTB5

8 – PTB4

PTA0 - 16

PTA1 - 15

PTA2 - 14

PTA3 - 13

RXD - 12

TXD - 11

PTB2 - 10

PTB3 - 9

Y+

Y-

X+

X-

Y- sense

X- sense

Y+ sense

X+ sense

RXD

TXD

Figure 5 – Touchscreen Connections to the QE8

gpio

- Sample Code Flow chart

Below is the flow chart for the main loop in the sample code.

Measure raw x and y coordinates

Run the raw coordinates thru the calibration routine

Has the Refresh timer expired?

Debounce the Pen-down pin and process any RX’d data.

Zero the Refresh timer

Is Pen-down detected?

Transmit coordinates

1st set of readings

Transmit previous coordinates

1st set of readings

Go to Sleep

Upon New Pen-Down Wakeup Signal

Y

N

Y

Y

N

N

N

Y

- Reference material

MC9S08QE8 Datasheet	……………………..	Freescale Website

MC9S08QE8 Reference Manual  …………………  Freescale Website

- Acronyms

ADC	………….. Analog Digital Converter

A/D	………….. Analog Digital Converter

COP	………….. Computer Operating Properly

GPIO	………….. General Purpose Input/Output

ISR	………….. Interrupt Service Routine

KBI	………….. Key Board Interrupt

NVM	………….. Non-Volatile Memory (Flash)

QG8	………….. Nickname for the MC9S08QG8 Microcontroller

SCI	………….. Serial Communication Interface – a UART port

Vdd	………….. Power Rail – typically 3.3 Volts

Vss	………….. Ground Rail

How to Reach Us:

Home Page:

www.freescale.com

E-mail:

support@freescale.com

USA/Europe or Locations Not Listed:

Freescale Semiconductor

Technical Information Center, CH370

1300 N. Alma School Road
Chandler, Arizona 85224
+1-800-521-6274 or +1-480-768-2130

support@freescale.com

Europe, Middle East, and Africa:

Freescale Halbleiter Deutschland GmbH

Technical Information Center

Schatzbogen 7

81829 Muenchen, Germany

+44 1296 380 456 (English)

+46 8 52200080 (English)

+49 89 92103 559 (German)

+33 1 69 35 48 48 (French)

support@freescale.com

Japan:

Freescale Semiconductor Japan Ltd.

Headquarters

ARCO Tower 15F

1-8-1, Shimo-Meguro, Meguro-ku,

Tokyo 153-0064, Japan

0120 191014 or +81 3 5437 9125

support.japan@freescale.com

Asia/Pacific:

Freescale Semiconductor Hong Kong Ltd.

Technical Information Center
2 Dai King Street
Tai Po Industrial Estate
Tai Po, N.T., Hong Kong
+800 2666 8080

support.asia@freescale.com

For Literature Requests Only:

Freescale Semiconductor Literature Distribution Center

P.O. Box 5405

Denver, Colorado 80217

1-800-441-2447 or 303-675-2140

Fax: 303-675-2150

LDCForFreescaleSeminconductor@hibbertgroup.com

Information in this document is provided solely to enable system and software implementers to use Freescale Semiconductor products. There are no express or implied copyright licenses granted hereunder to design or fabricate any integrated circuits or integrated circuits based on the information in this document.

Freescale Semiconductor reserves the right to make changes without further notice to any products herein. Freescale Semiconductor makes no warranty, representation or guarantee regarding the suitability of its products for any particular purpose, nor does Freescale Semiconductor assume any liability arising out of the application or use of any product or circuit, and specifically disclaims any and all liability, including without limitation consequential or incidental damages. “Typical” parameters that may be provided in Freescale Semiconductor data sheets and/or specifications can and do vary in different applications and actual performance may vary over time. All operating parameters, including “Typicals”, must be validated for each customer application by customer’s technical experts. Freescale Semiconductor does not convey any license under its patent rights nor the rights of others. Freescale Semiconductor products are not designed, intended, or authorized for use as components in systems intended for surgical implant into the body, or other applications intended to support or sustain life, or for any other application in which the failure of the Freescale Semiconductor product could create a situation where personal injury or death may occur. Should Buyer purchase or use Freescale Semiconductor products for any such unintended or unauthorized application, Buyer shall indemnify and hold Freescale Semiconductor and its officers, employees, subsidiaries, affiliates, and distributors harmless against all claims, costs, damages, and expenses, and reasonable attorney fees arising out of, directly or indirectly, any claim of personal injury or death associated with such unintended or unauthorized use, even if such claim alleges that Freescale Semiconductor was negligent regarding the design or manufacture of the part.

Freescale™ and the Freescale logo are trademarks of

Freescale Semiconductor, Inc. All other product or service names

are the property of their respective owners.

© Freescale Semiconductor, Inc. 2004. All rights reserved.

Error: Reference source not found

Error: Reference source not found

Error: Reference source not found