| TM | 07_5002_RN_ZIN05 |
| --- | --- |
|  | 3/13/2008 |
|  | 1.7 |

# MX27 VPU Decoder             WinCE 6.0 Release Notes

Abstract:

Release Notes for VPU Decoder

Keywords:

Multimedia codecs, H264, MPEG4, Directshow

Revision History

| Version | Date | Author | Change Description |
| --- | --- | --- | --- |
| 1.0 | 23-Jan-2007 | Raviraj Aithal | Initial Draft |
| 1.1 | 23-Feb-2007 | Raviraj Aithal | Added MPEG4 decode support |
| 1.2 | 13-Dec-2007 | Sukruth K | Porting of MX32 VPU Decoder onto MX27 Platform |
| 1.3 | 17-Dec-2007 | Lauren Post | Update for new release notes template |
| 1.4 | 21-Jan-2008 | Lauren Post | Update based on CPO comments |
| 1.5 | 04-Feb-2008 | Lauren Post | Update for WinCE 6.0 version |
| 1.6 | 28-Feb-2008 | Lauren Post | Updates for fixes and frame dropping |
| 1.7 | 13-Mar-2008 | Lauren Post | Updates for resolution |

Table of Contents

1	Introduction	4

1.1	Purpose	4

1.2	Scope	4

1.3	Audience Description	4

1.4	References	4

1.4.1	Standards	4

1.4.2	Freescale Multimedia References	4

1.5	Definitions, Acronyms, and Abbreviations	5

2	Release History	6

2.1	Assumptions and Known Problems	6

2.2	Support	6

3	List of Deliverables	7

3.1	Documentation	7

3.2	Public Headers	7

3.3	Binaries	7

3.4	DSHOW Source	7

3.5	Registry	7

4	Software Setup & Tools	8

5	Build Procedure	9

6	Test Application Execution	10

6.1	Test Bench for Decoder DSHOW filter	10

7	Pre compilation Options	11

7.1	DSHOW Options	11

7.2	DSHOW Test Bench Options	11

# Introduction

## Purpose

The purpose of this document is to provide information on the package contents, instructions on building library and test applications and test execution for MX27ADS with WinCE OS.

This section shall identify the overall purpose of this document.

## Scope

The scope is restricted to information on the package contents and instructions for building and testing. This document does not provide any details about the architecture or APIs in the Decoder.

This section shall identify the scope of this document. The scope may be a product or platform, one or more software architecture components, or one or more software features.

## Audience Description

The reader is expected to have basic understanding of Video Signal processing, H264 decoding, MPEG4 decoding and MS DirectShow Framework.

This section shall identify the audience of the document .If these people don’t constitute the distribution list of this document, precise this last.

## References

### Standards

- Draft ITU-T Recommendation and Final Draft International Standard of Joint Video Specification (ITU-T Rec. H.264 | ISO\IEC 14496-10 AVC), input.doc

- Proposed Draft AVC|H.264 Conformance Spec, JVT-J011.doc

- ITU-T H.263 video coding specification.

### Freescale Multimedia References

- VPU API Specification Document - c&m-tristanex-api_reference_manual-v1.3.0.doc

- VPU Decoder Data Sheet – mx27_vpu_dec_datasheet.pdf

- VPU Decoder Release Notes – mx27_vpu_dec_release_notes.pdf

This section shall list all documents referenced in this document. The references shall be numbered sequentially in brackets [Id] and list the document number and the document title. The document version or date and specific pages, sections and/or paragraphs should be noted if applicable.

## Definitions, Acronyms, and Abbreviations

| Term\Acronym | Definition |
| --- | --- |
| API | Application Programming Interface |
| AVC | Advanced Video Coding |
| FSL | Freescale |
| ISO | International Standards Organization |
| ITU | International Telecommunication Union |
| MPEG | Moving Pictures Expert Group |
| VPU | Video Processing Unit |
| DSHOW | Direct Show |

All non-standard terms, acronyms and abbreviations that are unique to this document should be included in this section. References to other appendices or reference documents may be included. Index information should be in bold.

- IEEE – Institute of Electrical and Electronics Engineers

- FRD – Feature Requirements Document

- SRS – System Requirements Specifications

- PRD – Product Requirements Document

Also see IEEE Std 1002-1987, IEEE Standard Taxonomy for Software Engineering Standards or refer to 99-9999-TYPE-SITE Glossary of terms.

# Release History

| Release number | Deliverables | features |
| --- | --- | --- |
| 1.0 | - Documentation - Application Interface header file - Decoder filter with sources and test application - Test vectors | - H264 and MPEG4 BaseLine Profile Decoder |
| 1.1 | - Same | - MPEG4 Decoder |
| 1.2 | - Same | - Porting of MX32 VPU Decoder onto MX27 platform |
| 1.4 | - Same | - WinCE 6.0 support |
| 1.5 | - Same | - WinCE 6.0 support fixes |
| 1.6 | - Same | - Frame dropping and trick modes – including test bench |
| 1.7 | - Same | - Fixes for odd resolutions |

- Table 1.  Details of the release

## Assumptions and Known Problems

- This filter supports H264 decoding and MPEG4 decoding only. The Filter will not be able to decode H264 streams less than 17 frames (as the re-ordering is enabled to meet conformance). This filter uses Post-Processor (eMMA on iMX27) for YUV to RGB565 conversion of the output data from the VPU Decoder. The Post processor supports only one instance and hence the decoder does not support multiple instances.

- This filter source is only supported on WinCE 6.0 using Intrinsic BSP

- Support for MP4 container content requires the FSL MP4 Parser filter provided in a separate package as well as the MX27 AAC Decoder package for audio support.

- Audio codecs not provided with this release.

## Support

If you have any questions or problems concerning this release, please contact a Freescale representative. Please include release version, board version, BSP version and any other relevant information.

# List of Deliverables

## Documentation

Base directory: \fsl_mm_wince\Multimedia\

| Subdirectory | Files |
| --- | --- |
| docs\ mx27_vpu_dec | mx27_vpu_dec_release_notes.pdf mx27_vpu_dec_datasheet.pdf |

## Public Headers

Base directory: \fsl_mm_wince\Multimedia\

| Subdirectory | File |
| --- | --- |
| API_include | FSLGuids.h |

## Binaries

Base directory: \fsl_mm_wince\Multimedia\

| Subdirectory | File |
| --- | --- |
| bin600\libarm9\ARMV4I\retail | WinCE 6.0 Decoder filter dll – mx27_vpu_dec_dshow.dll |
| bin600\exearm9\ARMV4I\retail | WinCE 6.0 Decoder Testbench - mx27_vpu_dec_dshow_testbench.exe |

## DSHOW Source

Base directory: \fsl_mm_wince\Multimedia\components\video\mx27_vpu_dec_dshow_6.0

| Subdirectory | Files |  |
| --- | --- | --- |
| dshow_filter |  | VPU Dshow decoder source and header files |
| dshow_test | VPU Dshow decoder test bench sources & header files |  |

## Registry

Base directory: \fsl_mm_wince\Multimedia\misc

| Subdirectory | Files |  |
| --- | --- | --- |
| misc |  | fslmm_mx27.reg for MX27 platforms |

# Software Setup & Tools

- Build machine should be running Microsoft WinXP

- Build machine should have following installed

  - Microsoft Windows CE 6.0 Visual Studio

  - WinCE BSP F14

- iMX27 ADS BSP must be built with VPU and eMMA drivers.

Location of VPU driver and eMMA driver in Catalog:

- Third Party->BSPs->Freescale MX27 ADS: ARMV4I ->Device Drivers->VPU

- Third Party->BSPs->Freescale MX27 ADS: ARMV4I ->Device Drivers->eMMA

- To support container format playback MP4 splitter & AVI filter needs to be added.

- Graphics and Multimedia Technologies  -> Media -> Media Formats->AVI Filter

-  Graphics and Multimedia Technologies  -> Media -> Media Formats-> Mpeg1   Parser Splitter

# Build Procedure

VPU Decoder would be available as Platform Builder dirs projects with the directory structure mentioned above.  The details of the projects are given below.

- mx27_vpu_dec_dshow_6.0 - This contains the DSHOW filter and the test bench (dshow_filter & dshow_test)

Steps to insert projects into PB workspace:

- Copy the directory Multimedia to the MX27ADS workspace.

- Right click on Subprojects in the MX27ADS BSP Solution Explorer, select the option ‘Add Existing Subproject….’.

- Select the option ‘Sources\Dirs Files(sources;dirs)’ in dropdown menu for “Files Of  Type”.

Open the “dirs” file in the folder \fsl_mm_wince\Multimedia\Components\Video\ mx27_vpu_dec_dshow_6.0

- (Visual Studio will automatically create  the mx27_vpu_dec_dshow_6.0.pbpxml).

Steps to build the DSHOW:

- Right click on  mx27_vpu_dec_dshow_6.0 present in the projects in Visual Studio solution

- Select “Rebuild”

This builds the vpu dshow filter (mx27_vpu_dec_dshow.dll) in  \fsl_mm_wince\Multimedia\bin600\ libarm9\ARMV4I \Retail  and test bench (mx27_vpu_dec_dshow_testbench.exe) in  \fsl_mm_wince\Multimedia\bin600\exearm9\

ARMV4I\Retail\.

# Test Application Execution

The package contains the test applications for

- Testing VPU Decoder DirectShow filter

## Test Bench for Decoder DSHOW filter

Update the project.reg (Parametric view in the PB) file with the  the registry file available in the folder “\\fsl_mm_wince\Multimedia\misc\fslmm_mx27.reg” before doing “Build Current BSP and Subprojects”.

Command for executing the test bench:

mx27_vpu_dec_dshow_testbench.exe < input filename > < Width > < Height > <decoder type>

Decoder type should either be “H264” or “MPEG4”

Command for executing the test bench and dumping the data to a file:

mx27_vpu_dec_dshow_testbench.exe < input filename > < output > < Width > < Height > <Decoder type>

For this rebuild the test application with FILE_SINK defined. Also in the decoder filter, disable PP.

NOTE:

1.The <output> here is a folder containing subfolders – y, cb and cr .

# Pre compilation Options

## DSHOW Options

The following C options need to be set

| C Defines | Purpose | Remarks |
| --- | --- | --- |
| PP | To enable the post processor. | When defined, this does the mirroring of the image & YUV to RGB conversion. |
| PERFORMANCE | To measure the performance | When defined, this measures the total time taken for decoding all the frames. |
| FRAME_DROPPING | Drops frames to maintain a/v sync | This has two types of frame dropping – drop at render and drop before decode – At Stopstreaming it will print out statistics of where drops occured and achieved frame rate |
| ENABLE_VPUROTATION | Rotates VPU for PP input | Removes extra memcpys to allow VPU output direct to PP input |
| _WINCE6_DDRAW | compilation | Allows compilation with fslguids.h |

## DSHOW Test Bench Options

The following C options need to be set

| C Defines | Purpose | Remarks |
| --- | --- | --- |
| FILE_SINK | To enable dumping the output to a file | If it is not defined, it renders using the default video renderer |