| TM | 07-5967-RN-ZIN05 |
| --- | --- |
|  | 3/14/2008 |
|  | 1.4 |

# MX27 VPU Encoder WinCE Release Notes

Abstract:

Release Notes for i.MX27 VPU Encoder

Keywords:

Multimedia codecs, H264, MPEG4, DirectShow, MX27

Revision History

| Version | Date | Author | Change Description |
| --- | --- | --- | --- |
| 0.1 | 29-Nov-2007 | Muralidhar | Initial Draft |
| 1.0 | 12-Dec-2007 | Lauren Post | Modifications based on new template |
| 1.1 | 21-Jan-2008 | Lauren Post | Updates based on CPO feedback |
| 1.2 | 04-Feb-2008 | Lauren Post | Updates for WinCE 6.0 |
| 1.3 | 14-Feb-2008 | Lauren Post | Updates for WinCE 6.0 |
| 1.4 | 28-Feb-2008 | Atif Saeed | Fix typos and grammatical errors, update input parameters, updates specific to WinCE6.0 |

Table of Contents

1	Introduction	4

1.1	Purpose	4

1.2	Scope	4

1.3	Audience Description	4

1.4	References	4

1.4.1	Standards	4

1.4.2	Freescale Multimedia References	4

1.5	Definitions, Acronyms, and Abbreviations	5

1.6	Document Location	5

2	Release History	6

2.1	Assumptions and Known Problems	7

2.2	Support	7

3	List of Deliverables	8

3.1	Documentation	8

3.2	Public Headers	8

3.3	Binaries	8

3.4	DSHOW Source	8

3.5	Registry	8

4	Software Setup & Tools	9

5	Build Procedure	10

6	Test Application Execution	11

6.1	Test Bench for Encoder DSHOW filter	11

7	Updating Registry	12

8	Pre compilation Options	13

8.1	DSHOW Options	13

8.2	Test Bench Options	13

9	APPENDIX	14

9.1	Build Procedure for regsvr32 utility	14

# Introduction

## Purpose

The purpose of this document is to provide information on the package contents, instructions on building the library and test applications and the test execution process for the MX27ADS on the WinCE operating system.

This section shall identify the overall purpose of this document.

## Scope

The scope is restricted to information on the package contents and instructions for building and testing. This document does not provide any details about the architecture or APIs in the Encoder.

This section shall identify the scope of this document. The scope may be a product or platform, one or more software architecture components, or one or more software features.

## Audience Description

The reader is expected to have basic understanding of video signal processing, H.264 encoding, MPEG4 encoding and MS DirectShow Framework.

This section shall identify the audience of the document .If these people don’t constitute the distribution list of this document, precise this last.

## References

### Standards

### Freescale Multimedia References

- VPU API Specification Document – c&m-tristanex-api_reference_manual_v1.3.0.doc

- VPU Data Sheet – mx27_vpu_datasheet.pdf

- VPU Encoder Release Notes– mx27_vpu_enc_release_notes.pdf

This section shall list all documents referenced in this document. The references shall be numbered sequentially in brackets [Id] and list the document number and the document title. The document version or date and specific pages, sections and/or paragraphs should be noted if applicable.

## Definitions, Acronyms, and Abbreviations

| Term\Acronym | Definition |
| --- | --- |
| API | Application Programming Interface |
| AVC | Advanced Video Coding |
| FSL | Freescale |
| ISO | International Standards Organization |
| ITU | International Telecommunication Union |
| MPEG | Moving Pictures Expert Group |
| VPU | Video Processing Unit |
| DSHOW | Direct Show |

All non-standard terms, acronyms and abbreviations that are unique to this document should be included in this section. References to other appendices or reference documents may be included. Index information should be in bold.

- IEEE – Institute of Electrical and Electronics Engineers

- FRD – Feature Requirements Document

- SRS – System Requirements Specifications

- PRD – Product Requirements Document

Also see IEEE Std 1002-1987, IEEE Standard Taxonomy for Software Engineering Standards or refer to 99-9999-TYPE-SITE Glossary of terms.

## Document Location

docs\mx27_vpu_enc

# Release History

| Release number | Deliverables | features |
| --- | --- | --- |
| 1.0 | - Documentation - Application Interface header file - Encoder filter with sources and test application - Registry | - Beta version of the Filter supporting the following - H264 Baseline Encoder - MPEG4 Simple Profile Encoder |
| 1.1 | - Same | - Documentation changes |
| 1.2 | - Same | - WinCE 6.0 support |
| 1.3 | - Same | - Fixes and cleanup |
| 1.4 | - Same | - Fixes and cleanup - Performance logging - Input parameter changes to test application - Documentation updates |
| 1.5 | - Same | - Source filter accepts camera as an input source |

- Table 1.  Details of the release

## Assumptions and Known Problems

- This filter supports H.264, MPEG4 and H.263 encoding only

- The filter has a dependency on the VPU driver and its APIs

- Filter works only on WinCE 6.0

- Test bench application requires the Freescale source filter which is provided in a separate package.

## Support

If you have any questions or problems concerning this release, please contact a Freescale representative. Please include the release version, the board version, the BSP version and any other relevant information.

# List of Deliverables

## Documentation

Base directory: \fsl_mm_wince\Multimedia\

| Subdirectory | Files |
| --- | --- |
| docs\mx27vpu_enc | mx27_vpu_enc_release_notes.pdf mx27_vpu_datasheet.pdf |

## Public Headers

Base directory: \fsl_mm_wince\Multimedia\

| Subdirectory | File |
| --- | --- |
| API_include | FSLGuids.h |

## Binaries

Base directory: \fsl_mm_wince\Multimedia\

| Subdirectory | File |
| --- | --- |
| bin500\libarm9\ARMV4I\retail | VPU Encoder filter – mx27_vpu_enc_dshow.dll |
| bin500\exearm9\ARMV4I\retail | Test Bench executable |

## DSHOW Source

Base directory: \fsl_mm_wince\Multimedia\components\video

| Subdirectory | Files |  |
| --- | --- | --- |
| mx27_vpu_enc_dshow_6.0 |  | PB Project file |
| mx27_vpu_enc_dshow_6.0\dshow_filter |  | VPU dshow dirs file , encoder source and header files |
| mx27_vpu_enc_dshow_6.0\dshow_test\ | VPU Dshow dirs file, test bench application header and source files |  |

## Registry

Base directory: \fsl_mm_wince\Multimedia\misc

| Subdirectory | Files |  |
| --- | --- | --- |
| misc |  | fslmm_mx27.reg for MX27 platforms |

# Software Setup & Tools

- Build machine should be running Microsoft WinXP

- Build machine should have following installed

  - Microsoft Windows CE 6.0

  - WinCE BSP F14

- i.MX27 ADS board should be running Freescale WinCE BSP F14. The BSP must be built with VPU drivers.

Location of VPU driver in Catalog:

- Third Party->BSPs->Freescale MX27 ADS: ARMV4I ->Device Drivers->Video Processing Unit

Note: If the Third Party is not visible then,

- Go to File -> Manage Catalog Items

- Click on Import…

- Choose mx27ads.cec

# Build Procedure

The VPU Encoder project can be loaded into Visual Studio 2005 is available as a  dirs projects with the directory structure mentioned above.  The details of the projects are given below.

- mx27_vpu_enc_dshow_6.0 - This contains the DirectShow filter and the test bench application.

Steps to insert projects into Visual Studio  workspace:

- Copy the directory ‘Multimedia’ to the MX27ADS workspace.

- Right click on projects in the MX27ADS BSP Solution Explorer, select the option ‘Add Existing Subproject….’.

- Select the option ‘Sources\Dirs Files (sources;dirs)’ in dropdown menu for “Files Of  Type”.

- Open the “dirs” file in the folder ...\Multimedia\components\video\ mx27_vpu_enc_dshow_6.0\ (PB will automatically create the mx27_vpu_enc_dshow_6.0.pbpxml).

Steps to build the DirectShow filter:

- Right click on dshow_filter icon in Visual Studio workspace under the mx27_vpu_enc folder

- Select “Build”

This builds the DirectShow (mx27_vpu_enc_dshow.dll) filter and the test bench (mx27_vpu_enc_dshow_testbench.exe) respectively in ...\Multimedia\bin\lib\ARMV4I\Retail\   and \Multimedia\bin\exe\ARMV4I\Retail\ in the same order.

# Test Application Execution

The package contains the test applications for

- Testing VPU Encoder DirectShow filter

## Test Bench for Encoder DSHOW filter

The VPU Encoder DirectShow filter and the FSL Source Filter need to be registered before using the test bench.   The utility for registering the dll is regsvr32.exe (for details regarding its build procedure refer to appendix)

Command for registering:

regsvr32.exe mx27_vpu_enc_dshow.dll

regsvr32.exe fsl_source_filter..dll

Command for executing the test bench:

For encoding:

mx27_vpu_enc_dshow_testbench.exe <INPUT> <OUTPUT> <width> <height> <bitrate in kbps> <target frame rate> <ENCODETYPE>, <key frame rate>

<INPUT> = <input folder name with y, cb & cr DIRECTORIES> or <raw (yuv) input file> or <CAMERA>

<OUTPUT> = <output file name> or <output folder name>

<CAMERA> = camera:<TIME>

<TIME> = camera timeout in seconds

<ENCODETYPE> = H264 or MPEG4 or H263

NOTE:

The <INPUT> here can be a folder containing subfolders – y, cb, cr,  a yuv input file or the camera. The source filter needs to be built with the respective #define to enable the folder or the single file option. The <bit rate> is in kbps and the <frame rate> is in milliseconds.

# Updating Registry

Following are the steps to update the registry settings in the registry:

- Copy the fslmm_mx27.reg to a location on your hard disk

- Locate platform.reg in the list of parameter files in the BSP

- Add the path of fslmm_mx27.reg as below in the platform.reg file

#include "<Path of file>\fslmm_mx27.reg"

- Select Build OS->Sysgen to build the BSP

The image built will have all the necessary registry settings for the Filter

# Pre compilation Options

## DSHOW Filter Options

The following C options need to be set

| C Defines | Purpose | Remarks |
| --- | --- | --- |
| PERFORMANCE | To measure the performance | When defined, this measures the total time taken for encoding the file. Two output files are generated which can be imported into an excel sheet for performance analysis. |
| VPU_ALLOCATOR_INTERFACE | Encoder Optimization | When defined, the filter avoids memcpy of the data on the input side. |

## Test Bench Options

The following C options need to be set

| C Defines | Purpose | Remarks |
| --- | --- | --- |
| FILE_SINK | To enable dumping the output to a file as oppose to rendering it on the display. | Rendering the output to the display has not been tested thoroughly. |
| OUTPUT_FILE | When enabled identifies <OUTPUT> to be the output file name. | When not defined the output filename is “encfile”. |

## Source Filter Options

The following C options need to be set

| C Defines | Purpose | Remarks |
| --- | --- | --- |
| SINGLE_FILE | Enables single yuv file input source or camera input | Disabling this define means that the input source is a directory containing folders for y, cb and cr data. Camera input is enabled only when this #define is set. |

# APPENDIX

## Build Procedure for regsvr32 utility

- Open the MX27ADS BSP Workspace in Visual Studio 2005

- Click on the Solution Explorer tab

- The REGSVR project can be found in the following location:

  - WINCE600 -> PUBLIC -> dcom -> sdk -> samples

- Right click on REGSVR and select Build

- The file regsvr32.exe will be available in one of the following folders :

  - % PBWORKSPACEROOT%\ RelDir\MX27_ARMV4I_Release for the release version

  - % PBWORKSPACEROOT %\RelDir\MX27_ARMV4I_Debug for the debug version.