T1040RDB CPLD Registers

Specification

Revision History

| Rev No. | Date | Description |
| --- | --- | --- |
| R1.0 | October 15, 2013 | first draft |

Register map

| Address | Name | Bit functions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | bit0 | bit1 | bit2 | bit3 | bit4 | bit5 | bit6 | bit7 |
| 00 | CHIPID1 | CHIPID1 |  |  |  |  |  |  |  |
| 01 | CHIPID2 | CHIPID2 |  |  |  |  |  |  |  |
| 02 | HWVER | HW_VER |  |  |  |  |  |  |  |
| 03 | SWVER | SW_VER |  |  |  |  |  |  |  |
| 10 | RSTCON1 | SW_RST | DDR_RST | EC1_RST | EC2_RST | SG_RST | QSG1_RST | QSG2_RST | XG_RST |
| 11 | RSTCON2 | - | - | - | - | TDMR_RST | PEX_RST | MPEX1_RST | MPEX2_RST |
| 12 | INTSR | THERM_INT | SG_INT | QSG1_INT | QSG2_INT | POTSA_INT | POTSB_INT | TDMR1_INT | TDMR2_INT |
| 13 | FLHCSR | BOOT_SEL | BANK_OR | SW_BANK_SEL0 | SW_BANK_SEL1 | SW_BANK_SEL2 | BANK_SEL0 | BANK_SEL1 | BANK_SEL2 |
| 14 | FANCSR | - | - | - | - | FAN_PWM |  |  |  |
| 15 | LEDCSR | STS_LED | FXS1_LED | FXS2_LED | FXS3_LED | FXS4_LED | FXO_LED | XG_LED1 | XG_LED2 |
| 16 | SFPCSR | - | - | - | - | SFP_DET | SFP_TXDIS | SFP_RXLOS | SFP_TXFAIL |
| 17 | MISCCSR | SPI_CS3_SEL | SLEEP_EN | REQ_MD |  | TDMR_PRS | PEX_PRS | T2081_DET | TEST_SEL_N |
| 18 | BOOTOR | - | - | - | - | - | - | - | BOOT_OR |
| 19 | BOOTCFG1 | rcw_src[0:7] |  |  |  |  |  |  |  |
| 1A | BOOTCFG2 | rcw_src8 | - | svr[0:1] |  | - | eng_use[0:2] |  |  |

Registers detail description

(1) Chip ID1 register (CHIPID1 – address 0x00) bit allocation

| Bit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol | CHIPID1 |  |  |  |  |  |  |  |
| Access | RO |  |  |  |  |  |  |  |
| Reset | 0x55 |  |  |  |  |  |  |  |

Note: RO – Read Only

(2) Chip ID2 register (CHIPID2 – address 0x01) bit allocation

| Bit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol | CHIPID2 |  |  |  |  |  |  |  |
| Access | RO |  |  |  |  |  |  |  |
| Reset | 0xAA |  |  |  |  |  |  |  |

(3)  Hardware version register (HWVER – address 0x02) bit allocation

| Bit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol | HW_VER |  |  |  |  |  |  |  |
| Access | RO |  |  |  |  |  |  |  |
| Reset | x |  |  |  |  |  |  |  |

Hardware version register (HWVER – address 0x02) bit description

| Bit | Symbol | Value | Description | Reset |
| --- | --- | --- | --- | --- |
| 7:0 | HW_VER |  | the version field of the hardware board. | x |

(4) Software version register (SWVER – address 0x03) bit allocation

| Bit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol | SW_VER |  |  |  |  |  |  |  |
| Access | RO |  |  |  |  |  |  |  |
| Reset | x |  |  |  |  |  |  |  |

Software version register (SWVER – address 0x03) bit description

| Bit | Symbol | Value | Description | Reset |
| --- | --- | --- | --- | --- |
| 7:0 | SW_VER |  | the version field of the CPLD software. | x |

(5) Reset control register (RSTCON – address 0x10) bit allocation

| Bit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol | SW_RST | DDR_RST | EC1_RST | EC2_RST | SG_RST | QSG1_RST | QSG2_RST | XG_RST |
| Access | WC | WC | WC | WC | WC | WC | WC | WC |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Note: WC – Write Clear

Reset control register (RSTCON1 – address 0x10) bit description

| Bit | Symbol | Value | Description | Reset |
| --- | --- | --- | --- | --- |
| 0 | SW_RST | 0 | No reset occurs | 0 |
|  |  | 1 | Write a logic 1 will produce whole board reset# signal, this bit can auto clear |  |
| 1 | DDR_RST | 0 | No reset occurs | 0 |
|  |  | 1 | Write a logic 1 will produce DDR3 reset# signal, this bit can auto clear |  |
| 2 | EC1_RST | 0 | No reset occurs | 0 |
|  |  | 1 | Write a logic 1 will produce RGMII PHY1(RTL82111E-VB) reset# signal, this bit can auto clear |  |
| 3 | EC2_RST | 0 | No reset occurs | 0 |
|  |  | 1 | Write a logic 1 will produce RGMII PHY2(RTL82111E-VB) reset# signal, this bit can auto clear |  |
| 4 | SG_RST | 0 | No reset occurs | 0 |
|  |  | 1 | Write a logic 1 will produce SGMII PHY(RTL82111DN) reset# signal, this bit can auto clear |  |
| 5 | QSG1_RST | 0 | No reset occurs | 0 |
|  |  | 1 | Write a logic 1 will produce QSGMII PHY1(VSC8514) reset# signal, this bit can auto clear |  |
| 6 | QSG2_RST | 0 | No reset occurs | 0 |
|  |  | 1 | Write a logic 1 will produce QSGMII PHY2(VSC8514) reset# signal, this bit can auto clear |  |
| 7 | XG_RST | 0 | No reset occurs | 0 |
|  |  | 1 | Write a logic 1 will produce 10G PHY(CS4315) reset# signal, this bit can auto clear |  |

(6) Reset control register (RSTCON2 – address 0x11) bit allocation

| Bit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol | - | - | - | - | TDMR_RST | PEX_RST | MPEX1_RST | MPEX2_RST |
| Access | - | - | - | - | - | WC | WC | WC |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Reset control register (RSTCON – address 0x11) bit description

| Bit | Symbol | Value | Description | Reset |
| --- | --- | --- | --- | --- |
| 0:3 | - |  |  | 0x0 |
| 4 | TDMR_RST | 0 | No reset occurs | 0 |
|  |  | 1 | Write a logic 1 will produce TDM riser card reset# signal, this bit can auto clear |  |
| 5 | PEX_RST | 0 | No reset occurs | 0 |
|  |  | 1 | Write a logic 1 will produce PCIe x4 slot reset# signal, this bit can auto clear |  |
| 6 | MPEX1_RST | 0 | No reset occurs | 0 |
|  |  | 1 | Write a logic 1 will produce miniPCIe card1 reset# signal, this bit can auto clear |  |
| 7 | MPEX2_RST | 0 | No reset occurs | 0 |
|  |  | 1 | Write a logic 1 will produce miniPCIe card2 reset# signal, this bit can auto clear |  |

(7) Interrupt status register (INTSR – address 0x12) bit allocation

| Bit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol | THERM_INT | SG_INT | QSG1_INT | QSG2_INT | POTSA_INT | POTSB_INT | TDMR1_INT | TDMR2_INT |
| Access | RO | RO | RO | RO | RO | RO | RO | RO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Interrupt status register (INTSR – address 0x12) bit description

| Bit | Symbol | Value | Description | Reset |
| --- | --- | --- | --- | --- |
| 0 | THERM_INT | 0 | No interrupt occurs | 0 |
|  |  | 1 | Board over temperature interrupt occurs |  |
| 1 | SG_INT | 0 | No interrupt occurs | 0 |
|  |  | 1 | SGMII PHY(RTL8211DN) interrupt occurs |  |
| 2 | QSG1_INT | 0 | No reset occurs | 0 |
|  |  | 1 | QSGMII PHY1(VSC8514) interrupt occurs |  |
| 3 | QSG2_INT | 0 | No reset occurs | 0 |
|  |  | 1 | QSGMII PHY2(VSC8514) interrupt occurs |  |
| 4 | POTSA_INT | 0 | No reset occurs | 0 |
|  |  | 1 | POTS A interrupt occurs |  |
| 5 | POTSB_INT | 0 | No reset occurs | 0 |
|  |  | 1 | POTS B interrupt occurs |  |
| 6 | TDMR1_INT | 0 | No reset occurs | 0 |
|  |  | 1 | TDM riser card interrupt 1 occurs |  |
| 7 | TDMR2_INT | 0 | No reset occurs | 0 |
|  |  | 1 | TDM riser card interrupt 2 occurs |  |

Note: INTSR register is related with system IRQ0(CPLD_INT1_N) signal, when interrupt occurs, IRQ0 will be logic 0 until all interrupts of INTSR register clear.

(8) Flash control and status register (FLHCSR – address 0x13) bit allocation

| Bit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol | BOOT_SEL | BANK_OR | SW_BANK_SEL0 | SW_BANK_SEL1 | SW_BANK_SEL2 | BANK_SEL0 | BANK_SEL1 | BANK_SEL2 |
| Access | RO | RW | RO | RO | RO | RW | RW | RW |
| Reset | x | 0 | x | x | x | 0 | 0 | 0 |

Note : RW - Read and Write

Flash control and status register (FLHCSR – address 0x13) bit description

| Bit | Symbol | Value | Description | Reset |
| --- | --- | --- | --- | --- |
| 0 | BOOT_SEL | 0 | Boot from 16bit NOR flash | x |
|  |  | 1 | Boot from 8bit NAND flash |  |
| 1 | BANK_OR | 0 | NOR flash bank select from CPLD override disable | 0 |
|  |  | 1 | NOR flash bank select from CPLD override enable |  |
| 2 | SW_BANK_SEL0 | 0 | NOR flash bank select bit0 of switch status is 0 | x |
|  |  | 1 | NOR flash bank select bit0 of switch status is 1 |  |
| 3 | SW_BANK_SEL1 | 0 | NOR flash bank select bit1 of switch status is 0 | x |
|  |  | 1 | NOR flash bank select bit1 of switch status is 1 |  |
| 4 | SW_BANK_SEL2 | 0 | NOR flash bank select bit2 of switch status is 0 | x |
|  |  | 1 | NOR flash bank select bit2 of switch status is 1 |  |
| 5 | BANK_SEL0 | 0 | NOR flash bank select bit0 set 0 | 0 |
|  |  | 1 | NOR flash bank select bit0 set 1 |  |
| 6 | BANK_SEL1 | 0 | NOR flash bank select bit1 set 0 | 0 |
|  |  | 1 | NOR flash bank select bit1 set 1 |  |
| 7 | BANK_SEL2 | 0 | NOR flash bank select bit2 set 0 | 0 |
|  |  | 1 | NOR flash bank select bit2 set 1 |  |

(9) Fan control and status register (FANCSR – address 0x14) bit allocation

| Bit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol | - | - | - | - | FAN_PWM |  |  |  |
| Access |  |  |  |  | RW |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |

Fan control and status register (FANCSR – address 0x14) bit description

| Bit | Symbol | Value | Description | Reset |
| --- | --- | --- | --- | --- |
| 0:3 | - |  |  | 0x0 |
| 4:7 | FAN_PWM | 0000 | PWM duty cycle is 0%, fan stop running | 1111 |
|  |  | 0001~1110 | PWM duty cycle is 6.7%~93.3%, fan speed control |  |
|  |  | 1111 | PWM duty cycle is 100%, fan full speed |  |

(10) Panel LED control and status register (LEDCSR – address 0x15) bit allocation

| Bit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol | STS_LED | FXS1_LED | FXS2_LED | FXS3_LED | FXS4_LED | FXO_LED | XG_LED1 | XG_LED2 |
| Access | RW | RW | RW | RW | RW | RW | RO | RO |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | x | x |

Panel LED control and status register (LEDCSR – address 0x15) bit description

| Bit | Symbol | Value | Description | Reset |
| --- | --- | --- | --- | --- |
| 0 | STS_LED | 0 | Panel STATUS LED off | 0 |
|  |  | 1 | Panel STATUS LED on |  |
| 1 | FXS1_LED | 0 | Panel FXS1 LED off | 0 |
|  |  | 1 | Panel FXS1 LED on |  |
| 2 | FXS2_LED | 0 | Panel FXS2 LED off | 0 |
|  |  | 1 | Panel FXS2 LED on |  |
| 3 | FXS3_LED | 0 | Panel FXS3 LED off | 0 |
|  |  | 1 | Panel FXS3 LED on |  |
| 4 | FXS4_LED | 0 | Panel FXS4 LED off | 0 |
|  |  | 1 | Panel FXS4 LED on |  |
| 5 | FXO_LED | 0 | Panel FXO LED off | 0 |
|  |  | 1 | Panel FXO LED on |  |
| 6 | XG_LED1 | 0 | Panel XG TX LED off when SFP+ not present or TX disabled | x |
|  |  | 1 | Panel XG TX LED on when SFP+ present and TX enabled |  |
| 7 | XG_LED2 | 0 | Panel XG RX LED off when SFP+ not present or RX los | x |
|  |  | 1 | Panel XG RX LED on when SFP+ present and RX optical received |  |

(11) SFP+ control and status register (SFPCSR – address 0x16) bit allocation

| Bit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol | - | - | - | - | SFP_DET | SFP_TXDIS | SFP_RXLOS | SFP_TXFAIL |
| Access |  |  |  |  | RO | RW | RO | RO |
| Reset | 0 | 0 | 0 | 0 | x | 0 | x | x |

SFP+ control and status register (SFPCSR – address 0x16) bit description

| Bit | Symbol | Value | Description | Reset |
| --- | --- | --- | --- | --- |
| 0:3 | - |  |  | 0x0 |
| 4 | SFP_DET | 0 | SFP+ module not inserted | x |
|  |  | 1 | SFP+ module inserted |  |
| 5 | SFP_TXDIS | 0 | SFP+ TX enable | 0 |
|  |  | 1 | SFP+ TX disable |  |
| 6 | SFP_RXLOS | 0 | SFP+ RX LOS logic 0 | x |
|  |  | 1 | SFP+ RX LOS logic 1(some SFP+ used as RXSD) |  |
| 7 | SFP_TXFAIL | 0 | SFP+ TX FAIL not occurs | x |
|  |  | 1 | SFP+ TX FAIL occurs |  |

(12) Miscellanies control and status register (MISCCSR – address 0x17) bit allocation

| Bit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol | SPI_CS3_SEL | SLEEP_EN | REQ_MD |  | TDMR_PRS | PEX_PRS | T2081_DET | TEST_SEL_N |
| Access | RW | RW | RW |  | RO | RO | RO | RO |
| Reset | 0 | 0 | 01 |  | 0 | x | x | x |

Miscellanies control and status register (MISCCSR – address 0x17) bit description

| Bit | Symbol |  | Value | Description |  | Reset |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | SPI_CS3_SEL |  | 0 | SPI_CS3 select TDMR SPI CS0 |  | 0 |
|  |  |  | 1 | SPI_CS3 select TDMR SPI CS1 |  |  |
| 1 | SLEEP_EN |  | 0 | Normal operation |  | 0 |
|  |  |  | 1 | Deep sleep enable bit Before enter deep sleep mode, set ‘1’ to this bit, after exit deep sleep mode, set ‘0’ to this bit |  |  |
| 2:3 | REQ_MD | 00 |  | No reset occurs when HRESET_REQ triggered | 0x01 |  |
|  |  | 01 |  | HRESET occurs when HRESET_REQ triggered |  |  |
|  |  | 10 |  | NA |  |  |
|  |  | 11 |  | PORESET occurs when HRESET_REQ triggered |  |  |
| 4 | TDMR_PRS |  | 0 | TDM riser card not present |  | x |
|  |  |  | 1 | TDM riser card present |  |  |
| 5 | PEX_PRS |  | 0 | PCIe x4 card not present |  | x |
|  |  |  | 1 | PCIe x4 card present |  |  |
| 6 | T2081_DET |  | 0 | T1040 on board |  | x |
|  |  |  | 1 | T2081 on board |  |  |
| 7 | TEST_SEL_N |  | 0 | TEST_SEL_N pin status is 0 |  | x |
|  |  |  | 1 | TEST_SEL_N pin status is 1 |  |  |

(13) Boot configuration override register (BOOTOR – address 0x18) bit allocation

| Bit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol | - | - | - | - | - | - | - | BOOT_OR |
| Access |  |  |  |  |  |  |  | RW |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Boot configuration override register (BOOTOR – address 0x18) bit description

| Bit | Symbol | Value | Description | Reset |
| --- | --- | --- | --- | --- |
| 7:1 | - |  |  | 0x0 |
| 0 | BOOT_OR | 0 | Boot configuration from CPLD override disable | 0 |
|  |  | 1 | Boot configuration from CPLD override enable |  |

(14) Boot configuration register 1 (BOOTCFG1 – address 0x19) bit allocation

| Bit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol | cfg_rcw_src[0:7] |  |  |  |  |  |  |  |
| Access | RW |  |  |  |  |  |  |  |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Note: for more information, refer to T1040 datasheet.

(15) Boot configuration register 2 (BOOTCFG2 – address 0x1A) bit allocation

| Bit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol | cfg_rcw_src8 | - | cfg_svr[0:1] |  | - | cfg_eng_use[0:2] |  |  |
| Access | RW |  | RW | RW |  | RW | RW | RW |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Note: for more information, refer to T1040 datasheet.